MUSHROOM STUDY: fMRI BIDS CONVERSION

LINUX SERVER

Remote desktop

fMRI data is way too large to process on your local computer, so we need to run all this on the institute’s servers. These servers run on an Ubuntu MATE Linux OS. For the remote desktop (to work on these servers) we use X2Go, for which a manual is stored in this folder as TUX_server_manual_X2Go.pdf. However, we might change to ThinLinc in the near future, with a similar way of logging in.

File transfers

Folders on the server can be connected to our project folder on fmgstorage, but it is not a very smooth process and it might potentially lead to a data overkill on the fmgstorage, so we will need to manually transfer files between fmgstorage (where our main data and code is stored) and the Linux server (where we need a copy of our MRI data and our analysis scripts).

On fmgstorage we keep our raw fmri data in raw_data/sub-xxx/scans, in which we keep our anatomical, functional and topup (for distortion correction) files per participant.

All this data needs to be copied from this fmgstorage folder to a folder on the server, for example to home/lhofman/Projects/mushroom_tux/raw_data.

From there, you can run the analyses (BIDS conversion, preprocessing, first-level, etc.) on the server, and you can copy important intermediate files back to the fmgstorage folder, as described in the respective paragraphs below.

Installation requirements

To run all steps for BIDS conversion, you need the following software installed:

- Python 3 (including pip3)

- Bidscoin

- dcm2niix (plugin for bidscoin)

MAIN SCRIPT

We keep our analysis scripts in our analysis folder on the server: home/lhofman/Projects/mushroom_tux/analysis. There, we have a subfolder for fMRI BIDS conversion: mushroom_tux/analysis/BIDS_conversion/scripts

The bash script bidsconversion.sh can be opened and run from the terminal, and incorporates the separate steps for BIDS conversion.

You can use one of the following options to run the script:

- Double-click on the file and select “Run in Terminal”

- Change directory in MATE terminal to file location  and type <bash bidsconversion.sh>

(or use full name, including directory and call from home directory)

To open and/or edit the script in RStudio: double click on the file and select “Display” or right-click on the file and select “Open in RStudio”.

BIDS CONVERSION

BIDS (Brain Imaging Data Structure; https://bids.neuroimaging.io/) is a specification on how to format, name, and organize your MRI dataset. It specifies the file format of MRI files (i.e., compressed Nifti: .nii.gz files), lays out rules for how you should name your files (i.e., with “key-value” pairs, such as: sub-001_task-mushroom_run-1_bold.nii.gz), and outlines the file/folder structure of your dataset (where each subject has its own directory with separate subdirectories for different MRI modalities, including fieldmaps, functional, diffusion, and anatomical MRI). Additionally, it specifies a way to include “metadata” about the (MRI) files in your dataset with JSON files: plain-text files with key-value pairs (in the form “parameter: value”). Given that your dataset is BIDS-formatted and contains the necessary metadata, you can use fmriprep on your dataset.

(The above paragraph was copy/pasted from Lukas Snoek’s tutorial on fMRI analysis.)

Below, I explain the different steps to take to correctly format our fMRI data.

Fix incomplete scans

Because our task included varying reaction times, and therefore varying times to complete a run, we had to stop our runs manually once the participant finished all trials. This has the unwanted consequence that the last volume might have been stopped halfway and is therefore incomplete. This needs to be fixed before ‘bidsifying’ our data.

The script fix_truncated_scans.py, which is called from the main bidsconversion.sh script, crops the .PAR header files (the files with all the parameters associated with the scan) of the scans with incomplete volumes by removing excess slices.

The original file with the incomplete volume will be kept in hidden view by renaming it to .<original_name_scan>_partialVolume.PAR (note the dot before the filename) before fixing the file.

How to call this function and which arguments to define can be found in the header of the fix_truncated_scans.py.

You can then find both the original and the fixed file in mushroom_tux/raw_data/sub-xxx/scans (make sure to toggle view>show hidden files to see the hidden files.

Bidsmapper

We will use the Bidscoin software (https://bidscoin.readthedocs.io/en/stable/), developed within the Donders Institute, to bidsify our data. The bidsmapper starts by building a map of the different kind of data types (scans) in your source dataset, which can then be edited with the bidseditor. The bidscoiner reads this so-called study bidsmap, which tells it how exactly to convert (“coin”) the source data into a BIDS data repository.

In its simplest form, bidsmapper scans your data in the specified source folder and creates a template study bidsmap. After the source data has been scanned, the bidsmapper will automatically launch the bidseditor to let you check and edit the automatically generated study bidsmap.

The first time I ran the bidsmapper and editor, it stored the bidsmap at mushroom_tux/bids/code/bidscoin/bidsmap.yaml. This bidsmap can be edited using the bidseditor, but also directly in the yaml file, or a combination of both.

Bidscoiner

Running the bidscoiner will then actually bidsify your data according to the prespecified bidsmap and store the bids data in mushroom_tux/bids/sub-xxx, with for each participant a separate folder for the anatomical scan, the functional scans and the fieldmaps. All scan data is now in nifti format rather than the original scanner format (PAR/REC) and metadata about these scans is stored in .json files.

To check if the bidscoiner did a good job, you run check_bids_conversion.py from the main preprocessing.sh script. This scripts check the number of bidsified mri files and compares it to the expected number of files (1 T1, 3 BOLD runds, 3 fieldmaps = 7 scan files; each scan file has an associated .json file; each subject has a .tsv file with an overview of all files: total nr of files = 15).

If the script outputs that there are missing files for a subject, please check which files are missing and determine if that makes sense (e.g. participant only completed 2 instead of 3 runs) or if the bidsmap is incomplete or misses scanmappings (in this case, edit the bidsmap and rerun the bidscoiner).

Add metadata to scan header file

Some metadata was not automatically stored during scanning, and needs to be added now: slice timing, effective echo spacing and total readout time. The add_metadata.py script, which can be called from the main bidsconversion.sh script, does just that for all .json files that belong to the functional or fieldmap scans that do not yet contain this information.

Create fieldmap images in both directions

Fieldmap images are used for distortion correction due to field inhomogeneity during scanning. The fieldmap images that we obtained during scanning are in the opposite direction (PA, j) of the main bold scans (AP, j-). However, the distortion correction step during fMRI preprocessing needs scans in both directions to compute the distortion and correct for it. Therefore, we already have 3 volumes in the opposite direction (PA, j; the original fieldmap scans), and we create 3 volumes in the scan direction (AP, j-) by copying the last 3 volumes from the bold runs and defining them as fieldmap volumes, and adding a corresponding .json file. For this, we use the script create_APPA_epi.py, which can be called from the main bidsconversion.sh script. The original and newly created fieldmap images and their .json files are then renamed such that the phase encoding direction is included in their filename, and stored in mushroom_tux/bids/sub-xxx/fmap.